__________________________________________________

Installation Information For Windows 9x/ME
ATI Enhanced Display Driver

__________________________________________________

September 2001

__________________________________________________

READ THIS FILE COMPLETELY BEFORE ATTEMPTING TO USE THIS PRODUCT!

__________________________________________________

Contents

1.0 Introduction

2.0 Adapter Installation

3.0 Installation Notes For Accelerated Graphics Port (AGP) Systems

4.0 Installation Notes For Windows 98/ME ATI Enhanced Display Driver

5.0 More Documentation

__________________________________________________________________________________

1.0 Introduction

This file contains important installation notes for ATI enhanced display driver. The file also contains updates that are not found in the Online User's Manual or in any addendum accompanying the display adapter.

__________________________________________________________________________________

2.0 Adapter Installation

Refer to the Online User's Manual for information on adapter installation.

__________________________________________________________________________________

3.0 Installation Notes For Accelerated Graphics Port (AGP) Systems

If you are installing an ATI AGP graphics adapter, your system may require installation of the most recent AGP services VxD (virtual device driver), in addition to the ATI enhanced display drivers.

AGP supports a Direct Memory Execute (DIME) feature, which allows AGP graphics cards to use system memory (typically for storing large, detailed texture bitmaps) as if it were on-board graphics memory.  A device called a Graphics Address Remapping Table (GART), under the control of a VGARTD VxD driver, manages these system memory addresses ("VGARTD" stands for Virtual GART Driver).

FOR SYSTEMS USING INTEL CHIPSETS

Windows 95 users must be running OEM Service Release (OSR) version 2.1, or later, and have VGARTD.VxD installed (in the Systems folder); otherwise AGP functionality will not be available

The program USBSUPP.EXE (supplied with Windows 95 OSR 2.1) is also required, as this installs the VMM32.VxD memory manager that is needed by AGP's DIME feature. DirectX 5 or greater is needed to support AGP's DIME feature.

Windows 98 natively provides this support as long as DirectX 5.0 (or greater) is installed.  You may, however, require an updated VGARTD.VxD to support newer Intel chipsets.  Your system vendor should have included the appropriate GART file as part of you Windows install image.  However, if you experience display problems following installation, you should obtain the latest Virtual GART (AGP) Driver from the motherboard manufacturer. If you cannot locate the motherboard manufacturer, generic VxD drivers can usually be obtained from the Intel web site. http://developer.intel.com/design/chipsets/

FOR SYSTEMS USING NON-INTEL CHIPSETS

Several AGP motherboard manufacturers use non-Intel AGP chipsets. Chipsets include those made by Acer Laboratories (ALI), Silicon Integrated Systems (SIS), and VIA Technologies, Inc. Each non-Intel chipset requires the installation of a custom Virtual GART Driver. It is very important that the correct VxD driver be installed before operating your system.  An incorrect or missing GART can result in no AGP memory being detected or a black screen after Windows loads.

In most cases, for those chipset manufacturers just mentioned, the system vendor will have included the appropriate GART file as part of you Windows install image.   However, if you experience display problems following installation, you should obtain the latest Virtual GART (AGP) Driver from the motherboard manufacturer. If you cannot locate the motherboard manufacturer, generic VxD drivers can usually be obtained from the chipset manufacturer.

Please refer to one of the following websites:

Motherboard/chipset information at http://www.motherboards.org

Acer Laboratories (ALI) at http://www.ali.com.tw/eng/support/driver.shtml

Silicon Integrated Systems (SIS) at http://www.sis.com.tw/driver/driver.htm

VIA Technologies at http://www.via.com.tw/drivers/index.htm

GARTS PROVIDED ON THE CD

ATI has provided, on this CD, the most current and common GART files that were available at the time of release.  They are located on the ATI CD under the subdirectory \INSTALL\GART.

The GART files may be out of date and newer ones may be found at manufacturers web site.

__________________________________________________________________________________

4.0 Installation Notes For Windows 9x/ME ATI Enhanced Display Driver

Note: Before installing your ATI enhanced drivers, make sure that any previously installed ATI display drivers have been removed from your system. This can be done using the Add/Remove Programs option in the Control Panel.

After the installation of your ATI graphics accelerator card, new hardware may be detected after restarting your computer.  Follow the step-by-step instructions below to allow Windows to correctly identify your new hardware.

4.1 To identify new hardware in Windows 9x/ME

1. After installing the graphics adapter and starting Windows 95, the Update Device Driver Wizard will launch.  If the Wizard does not appear, follow the instructions under 4. 5 Installing the ATI Enhanced Drivers for Windows 98 and Windows ME.

2. Click Next.

3. Click Finish.

4. Insert your Windows 98 or Windows ME CD-ROM into your CD-ROM drive.

5. Click OK.

6. Type the following:

D:\WIN95

(If D is not your CD-ROM drive, then substitute D with the correct drive letter.)

7. Click OK.

8. Click Yes to restart your computer.

You are now ready to install your ATI enhanced drivers.  Proceed to 4.5 Installing the ATI Enhanced Drivers for Windows 95 and Windows 98 below, and follow the instructions to complete the installation.

4.2 To identify new hardware in Windows 98/ME

After restarting your system, Windows 98/ME will briefly display the "New Hardware Found" dialog. Depending on your new hardware, you will either be prompted to insert your Windows 98 CD-ROM or Windows 98/ME will launch the Add New Hardware Wizard.

If Windows 98 automatically begins installing drivers for your new hardware, click Yes to restart your computer when prompted; then proceed to 4.5 Installing the ATI Enhanced Drivers for Windows 95 and Windows 98 below.

4.3 To identify new hardware using the Windows 98 CD-ROM

1. Insert your Windows 98 CD-ROM.

2. Click OK.

3. Type the following:

D:\WIN98

(If D is not your CD-ROM drive, substitute D with the correct drive letter.)

4. Click OK.

5. Click Yes to restart your computer.

You are now ready to install your ATI enhanced drivers.  Proceed to 4.5 Installing the ATI Enhanced Drivers for Windows 95 and Windows 98 below, and follow the instructions to complete the installation.

4.4 To identify new hardware using the "Add New Hardware Wizard" in Windows 98

1. After installing the graphics adapter and starting Windows 98, the Add New Hardware Wizard will prompt for a “Standard PCI Graphics Adapter (VGA)” display driver.

2. Click Next to continue.

3. Select Search for the best driver for your device.

4. Click Next to continue.

5. Click Next to start the driver search.

6. Click Next to install the Standard PCI Graphics Adapter (VGA).

7. Click Finish.

8. Click Yes to restart your computer.

You are now ready to install your ATI enhanced drivers. Proceed to the section "Installing Enhanced Drivers for Windows 95 and Windows 98" below, and follow the step-by-step instructions to complete the installation.

4.5 Installing the ATI Enhanced Drivers for Windows 9x/ME

With Windows 98 or Windows ME running on your computer, you need to install the enhanced ATI drivers to take advantage of your card's higher performance, resolutions, and special features.

To ensure that you install the latest drivers, install the ATI enhanced drivers located on the CD-ROM shipped with your ATI graphics accelerator card.

1. Insert the ATI INSTALLATION CD-ROM into your CD-ROM drive.

If Windows runs the CD-ROM automatically, proceed to step6.

2. Click Start.

3. Select Run.

4. Type the following:

D:\ATISETUP

(If D is not your CD-ROM drive, then substitute D with the correct drive letter.)

5. Click OK.

6. Click on ATI Easy Install to begin the Installation Wizard.

7. Click Next.

8. Click Yes.

9. Follow the Wizard's on-screen instructions to complete the installation.

The Express installation option is recommended. If your ATI product includes a multimedia component, the software for that component will automatically be installed, along with the ATI enhanced drivers, by selecting this option.

4.6 Installing the Windows 95/98 ATI display drivers via "Add/Remove Programs"

1. Start Windows 95/98.

2. Insert the ATI INSTALLATION CD-ROM into your CD-ROM drive.

3. Click Start.

4. Point to Settings, then click Control Panel.

5. Double-click Add/Remove Programs.

The "Add/Remove Programs Properties" dialog appears.

6. Click the Install/Uninstall tab.

7. Click the Install button.

The "Install Program From Floppy Disk or CD-ROM" dialog appears.

8. Click Next to continue.

The "Run Installation Program" dialog appears.

9. Type the following:

D:\ATISETUP

(If D is not your CD-ROM drive, then substitute D with the correct drive letter.)

10. Click OK.

11. Click on ATI Easy Install to begin the Installation Wizard.

12. Follow the Wizard's on-screen instructions to complete the installation.

__________________________________________________________________________________

5.0 More Documentation

More files are located on this CD in these locations:

doc/Instl_NT.doc

doc/Instl_9x.doc

doc/Instl_2k.doc

doc/Remov_NT.doc

doc/Remov_9x.doc

doc/Remov_2k.doc

ATI Multimedia Center supported products, please refer to:

\DOC\MMCENTER\MMC7.wri