_____________________________________________________________________________________

Important Information for RADEON and RADEON VE Windows 9x Enhanced Display Driver

___________________________________________________

January 2001

_____________________________________________________________________________________

READ THIS FILE COMPLETELY BEFORE ATTEMPTING TO USE THIS PRODUCT!

___________________________________________________

Contents

Introduction

Windows 9x ATI RADEON enhanced display driver known issues

Windows 9x Issues

Application Issues

_____________________________________________________________________________________

Introduction

This file contains last-minute updates that are not found in the Online User's Manual or in any addendum accompanying the display adapter.

A number of problems are known to exist with this release.  This readme highlights a number of these issues and, where possible, presents workarounds until complete solutions can be implemented. Please read this file before proceeding with the installation.

This driver supports Microsoft Windows 95 and Microsoft Windows 98

____________________________________________________________________________________

Windows 9x ATI RADEON enhanced display driver known issues

- Hardware acceleration is not supported for 8bpp and 24bpp in Direct3D and OpenGL.

- On some older systems using the AWARD system BIOS, there is a limitation which restrict the video BIOS to be no larger than 32K.  To support the extended TV Out feature on the RADEON video cards, the video BIOS is larger than 32K.  The effect of this limitation by the system BIOS will cause a black screen anomaly when the TV is enabled.  To resolve this problem, it is advisable that you contact your system manufacturer to update the system BIOS.

- _____________________________________________________________________________________

- Windows 9x Issues

- At 8bpp color depth (any resolution), playing back two AVI files simultaneously can cause color corruption in the AVI file without overlay. This is a design limitation of 8bpp that by definition uses a palette.  Windows supports only one active palette.

- A solution to this problem: choose a color depth other than 8bpp, or only play a single AVI file at a time.

- On AMD-K6/2-300 systems using the SIS 5591 chipset, there is a known problem whereby following restart, DX Info viewer reports that there is no AGP memory, even with the latest SIS GART file installed.

- There are known issues with certain ViewSonic monitors, including models G810 and V775, whereby selecting a Plug & Play monitor results in the loss of particular mode selections.

- A solution to this problem: rather than selecting a Plug & Play monitor, install the monitor using the Monitor.inf file which came with your monitor. If this file is unavailable, you can download VSINF.exe from the monitor manufacturer (ViewSonic) at:

- http://www.viewsonic.com/DESK/DESK.HTM

- When running "Organic Art" screen saver with multiple monitors you may experience a hang.  This problem is not limited to ATI cards and the Microsoft Support website has addressed several problems with the Organic Art screen saver when running dual-monitor configuration.

A solution to this problem (from Microsoft Support website): Edit the registry to configure your screen saver to run on only one monitor.

To do this, add a new DWORD value named "DisableMultiMon" (without quotation marks) with value data of "1" (without quotation marks) to the following registry key:

HKEY_CURRENT_USER\Software\Computer Artworks\Organic Art\Plus

_____________________________________________________________________________________

Application Issues

- To play Microsoft Baseball 2000, compressed texture formats must be disabled. This is required since the game cannot handle a large number of supported texture formats.  To disable the compressed texture formats, please add a registry value (a string) "ExportCompressedTex" in registry key [HKEY_LOCAL_MACHINE\Software\ATI Technologies\Driver\000x\atidxhal/    (000x is a display ID like 0001, 0002, ...) and set the registry value data to 0.